Table 1-1  Fermentation reactions examples

| Product | Reaction | G0’(kJ/mole) |
| --- | --- | --- |
| Lactate | C6H12O6 2 CH3CH(OH)COO- + 2 H+ | -198.1 |
| Butyrate | C6H12O6 + 2H2O CH32(CH2)COO- + 2 HCO3- +2 H2 + 3H+ | -254.4 |
| Propionate + Acetate | 1.5 C6H12O6 2 CH3CH2COO- + CH3COO- + HCO3- + 4H+ | -109.9 |

Table 1-2 Acetogenic reactions examples

| Substrate | Reaction | G0’ (kJ/mole) |
| --- | --- | --- |
| Lactate | CH3CH(OH)COO- + 2 H2O CH3COO- + HCO3- + 2H2 + H+ | -3.96 |
| Butyrate | (CH3)2(CH2)COO- + 2 H2O 2 CH3COO- + 2 H2 + H+ | +48.1 |
| Propionate | CH3CH2COO- + 3 H2O CH3COO- + HCO3- + 3 H2 + H+ | +76.1 |

Table 1-3 – Different types of methane-forming microorganism

| Substrate | Substrate utilization reaction | Archaea |
| --- | --- | --- |
| Hydrogen | CO2 + 4 H2 + H+CH4 + 3 H2O | Methanobacteriales, Methanococcales, Methanomicrobiales, Methanopyrales |
| Acetic acid | CH3COOH CH4 + CO2 | Methanosarcinales |
| Formate | 4 HCOO- + 4 H+ CH4 + 3 CO2 + 2 H2O | Methanobacteriales, Methanomicrobiales, Methanococcales |
| Methanol | 4 CH3OH 3 CH4 + CO2 + 2 H2O | Methanosarcinales |
| Methylamines | 4 (CH3)3N + H2O 9 CH4 + 3 CO2 + 6 H2O + 4 NH3 | Methanosarcinales |

Table 1-4 – Fixed film reactors treating industrial wastewaters

| Wastewater treated | Packing material | OLR (kg COD/m3 d) | CODrem (%) | HRT (d) | Methane Yield (m3CH4/kg CODrem) | References |
| --- | --- | --- | --- | --- | --- | --- |
| Distillery | Charcoal, coconut coir and nylon | 23.3 | 64 | 8 | 0.31 | Acharya et al. (2008) |
| Bulk drug industry | PVC rings | 10 | 60 - 70 | 1.7 | 0.25 | Gangagni Rao et al. (2005) |
| Distillery | Charcoal | 8 - 19 | 80 - 51 | 8 | - | Acharya et al. (2011) |
| Sugary | PVC pipe pieces | 3.0 | 82 | 0.5 | - | Pradeep et al. (2014) |

Table 1-5 – UASB reactors treating different industrial wastewaters

| Wastewater treated | OLR (kg COD/m3d) | CODrem (%) | HRT (h) | Methane Yield (m3CH4/kg CODrem) | References |
| --- | --- | --- | --- | --- | --- |
| Dairy | 28 | > 90 | 48 | - | Kalyuzhnyi et al. (1997) |
| Sugary | 4 | > 90 | 43 | 0.34 | Mijaylova Nacheva et al. (2009) |
|  | 24 | 80 | 7 | 0.35 |  |
| Textile | 6 | > 90 | 24 | - | Somasiri et al. (2008) |
| Brewery | 3 | 85 | 12 | 0.34 | Cronin and Lo (1998) |

Table 1-6 – AFR treating different industrial wastewaters

| Wastewater treated | Carriers type | OLR (kg COD/ m3 d) | CODrem (%) | HRT (d) | Methane Yield (m3CH4/kg CODrem) | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| winery distillery (IFBR) | perlite | 4.5* | 85* | 1.5 | - | Garcia-Calderon et al. (1998) |
| Brewery (IFBR) | triturated polyethylene | 10 | < 90 | 4 | 0.35 | Alvarado-Lassman et al. (2008) |
|  | Silica | 30 | < 90 | 4 |  |  |
| petrochemical | bone charcoal | 21.7 | 95 | 2.5 | 0.45 | Patel and Madamwar (2001) |
|  |  | 20.4 | 95 | 4 | 0.37 |  |

IFBR – Inverse Fluidized Bed Reactor; * as Total Organic Carbon (TOC).

Table 1-7 – Anaerobic baffled reactor treating different industrial wastewaters

| Wastewater treated | OLR (kg COD/m3 d) | CODrem (%) | HRT (d) | Methane Yield (m3CH4/kg CODrem) | References |
| --- | --- | --- | --- | --- | --- |
| Brewery | 5.6 | 90 | 15 | - | Hassan and Dahlan (2013) |
| Pulp and Paper | 5 | 60 | 48 | 0.18 | Grover et al. (1999) |
| Palm Oil Mill | 1.6 | 77 | 3 | 0.33 | Faisal and Unno (2001) |

Table 1-8 – Anaerobic hybrid reactor treating different wastewaters

| Reactor Type | Wastewater treated | OLR (kg COD/m3d) | CODrem (%) | HRT (d) | Methane Yield (m3CH4/kg CODrem) | References |
| --- | --- | --- | --- | --- | --- | --- |
| AF-UASB | distillery | 8.7 | 80 | 5 | 0.34 | Sunil Kumar et al. (2007) |
| UASFF | yeast | 9 | 75 | 3 | 0.32 | Büyükkamaci & Filibeli (2002) |
| UASB-MBBR | fruit-juice | 6 | 65 | 0.8 | 0.33 | Tawfik & El-Kamah, (2012) |
|  | winery | 25 | > 80 | 0.75 | - | Wahab et al. (2014) |
| UASB-AFR | synthetic* | 6 | 75 | 1.7 | 0.21 | Kundu, Sharma, & Sreekrishnan (2013) |

*glucose and yeast; AF-UASB: anaerobic filter – upflow anaerobic sludge blanket reactor; UASF: upflow anaerobic sludge bed fixed film reactor; UASB-MBBR: upflow anaerobic sludge blanket – moving bed biofilm reactor; UASB-AFR: upflow anaerobic sludge blanket – anaerobic fluidized bed.

Table 1-9 – Different type of media and specific surface area

Table 1-10 – AMBBR treating industrial wastewater

Table 1-11 – Typical brewery wastewater characteristics (Chastain et al., 2011)

| Parameter | Typical Range | Unit |
| --- | --- | --- |
| Water to beer ratio | 4 – 10 | L/L |
| Wastewater to beer ratio | 1.3 – 3.5 | L/L |
| BOD5 | 0.6 – 5 | kg/m3 |
| COD | 1.8 – 5.5 | kg/m3 |
| Total Nitrogen | 0.03 – 0.1 | kg/m3 |
| Total Phosphorus | 0.03 – 0.1 | kg/m3 |
| pH | 3 – 12 | - |
| Total suspended solids | 0.2 – 1.5 | kg/m3 |

Table 1-12 – Requirement from City of Winnipeg sewer by-law No.92/2010

| Parameter | Threshold | Unit |
| --- | --- | --- |
| TSS | 0.35 | kg/m3 |
| BOD5 | 0.3 | kg/m3 |
| TN | 0.06 | kg/m3 |
| TP | 0.01 | kg/m3 |
| Animal or vegetable oil and grease | 0.1 | kg/m3 |
| Mineral or synthetic oil and grease | 0.15 | kg/m3 |
| pH | 5.5 – 11 | - |

Table 2-1 – Micronutrient concentrations

| Micronutrient | Value | Units |
| --- | --- | --- |
| FeCl2•4H2O | 1.5 | mg/L |
| MnSO4•H2O | 0.085 | mg/L |
| CaCl2 | 0.18 | mg/L |
| MgSO4•H2O | 0.69 | mg/L |
| CoCl2•6H2O | 0.48 | mg/L |
| H3BO3 | 0.006 | mg/L |
| ZnSO4•7H2O | 0.14 | mg/L |
| CuSO4•5H2O | 0.0025 | mg/L |
| Na2MoO4•2H2O | 0.556 | mg/L |
| NiCl2•6H2O | 2.38 | mg/L |
| Na2SeO3•5H2O | 0.263 | mg/L |

Table 2-2 - Brewery wastewater characterization

| Parameter | Value | Unit |
| --- | --- | --- |
| sCOD | 5.8 ± 2.2 | kg sCOD/m3 |
| COD | 6.9 ± 2.7 | kg TCOD/m3 |
| BOD5 | 5.3 ± 0.2 | kg BOD5/m3 |
| BOD5/COD | 0.77 | - |
| TSS | 0.35 ± 0.12 | kg TSS/m3 |
| VSS | 0.3 ± 0.1 | kg VSS/m3 |

BOD5 is the 5 days Biological Oxygen Demand; sCOD is the soluble Chemical Oxygen Demand; COD is the total Chemical Oxygen Demand; TSS is the Total Suspended Solids and VSS are the Volatile Suspended Solids.

Table 3-1 – AMMBR influent and effluent characteristics for the entire study

|  | Parameter | Minimum | Maximum | Average | Standard Deviation | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| Influent | pH | 6.4 | 7.1 | 6.7 | 0.1 | - |
|  | TSS | 0.1 | 3.6 | 0.5 | 0.6 | kg TSS/m3 |
|  | VSS | 0.1 | 2.3 | 0.4 | 0.4 | kg VSS/m3 |
|  | sCOD | 2.6 | 5.9 | 3.8 | 1.0 | kg sCOD/m3 |
|  | TCOD | 2.8 | 9.2 | 5.0 | 1.6 | kg TCOD/m3 |
|  | BOD5 | 2.1 | 7.1 | 3.8 | 1.2 | kg BOD5/m3 |
| Effluent | pH | 6.7 | 7.3 | 7.0 | 0.1 | - |
|  | Alkalinity | 1.3 | 2.0 | 1.6 | 0.1 | kg CaCO3/m3 |
|  | TSS | 0.1 | 0.8 | 0.3 | 0.2 | kg TSS/m3 |
|  | VSS | 0.1 | 0.8 | 0.3 | 0.1 | kg VSS/m3 |
|  | sCOD | 0.1 | 1.8 | 0.7 | 0.5 | kg sCOD/m3 |
|  | TCOD | 0.3 | 2.6 | 1.1 | 0.6 | kg TCOD/m3 |
|  | BOD5 | 0.2 | 2.0 | 0.9 | 0.5 | kg BOD5/m3 |

Results presented neglecting the first week after a particular change of HRT was made.

Table 3-2 – Review of AMBBR processes treating industrial wastewater

| Wastewater treated | Carrier |  | sCOD rem (%) | SALR (g sCOD/m2 d) | Reference |
| --- | --- | --- | --- | --- | --- |
|  | Type | SAA (m2/m3) |  |  |  |
| Wine distillery | R30* | 320 | 85 | 87 | Sheli et al. (2010) |
| Wine distillery | R9* | 530 | 80 | 85 | Sheli et al. (2014) |
| Dairy | R9* | 530 | 85 | 58 | Wang et al. (2009) |
| Brewery | AC920 | 680 | 70 | 87 | This study |
|  |  |  | 80 | 56 |  |
|  |  |  | 85 | 40 |  |
|  |  |  | 90 | 24 |  |

* Bioflow manufactured by Raushert Co.

Table 4-1 – Beer production and wastewater generation rates

| Size | Beer Production (m3/d) | Wastewater : beer produced (m3/d) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 2 : 1 | 4 : 1 | 10 : 1 |
| Small | 3 | 5 | 11 | 27 |
| Medium | 137 | 274 | 548 | 1370 |
| Large | 411 | 822 | 1644 | 4110 |

Table 4-2 – Mass of COD produced in different breweries size

| Size | Wastewater : beer produced (kg COD/d) |  |  |
| --- | --- | --- | --- |
|  | 2 : 1 | 4 : 1 | 10 : 1 |
| Small | 30 | 40 | 49 |
| Medium | 1507 | 2000 | 2466 |
| Large | 4521 | 6000 | 7397 |

Table 4-3 – Surface area loading rates at particular sCOD percentage removal

| sCOD % removal | SALR (g sCOD m3/d) |
| --- | --- |
| 80 | 69 |
| 85 | 46 |
| 90 | 23 |

Table 4-4 – Total surface area required to achieve COD 80 and 90 % removals

|  | > 80 COD rem |  |  | > 90 COD rem |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Wastewater : beer production |  |  | Wastewater : beer production |  |  |
| Brewery size | 2 : 1 | 4 : 1 | 10 : 1 | 2 : 1 | 4 : 1 | 10 : 1 |
| Small (m2) | 542 | 719 | 887 | 1245 | 1653 | 2038 |
| Medium (m2) | 27102 | 35971 | 44348 | 62267 | 82645 | 101891 |
| Large (m2) | 81305 | 107914 | 133044 | 186800 | 247934 | 305672 |

Table 4-5 – Media volume (m3) required for the three types of media selected

| Size | Wastewater per beer ratio | > 80 COD rem |  |  | > 90 COD rem |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AC450 | AC515 | AC920 | AC450 | AC515 | AC920 |
| Small | 2 : 1 | 1.3 | 1.1 | 0.8 | 3.1 | 2.6 | 1.8 |
|  | 4 : 1 | 1.8 | 1.5 | 1.1 | 4.1 | 3.4 | 2.4 |
|  | 10 : 1 | 2.2 | 1.8 | 1.3 | 5.1 | 4.2 | 3.0 |
| Medium | 2 : 1 | 67 | 56 | 39.9 | 155 | 128 | 92 |
|  | 4 : 1 | 89 | 74 | 52.9 | 206 | 170 | 122 |
|  | 10 : 1 | 110 | 91 | 65.2 | 253 | 210 | 150 |
| Large | 2 : 1 | 202 | 168 | 119.6 | 465 | 385 | 275 |
|  | 4 : 1 | 268 | 223 | 158.7 | 617 | 511 | 365 |
|  | 10 : 1 | 331 | 274 | 195.7 | 760 | 630 | 450 |

Table 4-6 – Design parameters for AC920 media at 70% fillings

| sCOD removal (%) | Total surface area required (m2) | Volume media required (m3) | Reactor volume (m3) | Effluent concentration (gBOD5/L) |
| --- | --- | --- | --- | --- |
| 85 | 19500 | 29 | 41 | 0.675 |
| 90 | 39100 | 58 | 82 | 0.45 |